### DOOR HEATERS

In the refrigeration industry, a large number of commodities and food products are stored at sub-zero temperatures to prevent deterioration and maintain product quality. Cold-room storage facilities typically operate at temperatures ranging from -20°C to -40°C. Under these conditions, when atmospheric moisture comes into contact with the cold air around the door frames, ice formation can occur. This may cause the doors to jam and affect the proper functioning of the cold room.

Door Heater Tapes are designed to prevent ice formation around cold-room door frames. The heater tapes are installed in specially designed grooves provided along the door frames, where they generate controlled heat to prevent frost and ice accumulation.

| Sr. No. | Specification | Details |
| --- | --- | --- |
| 1 | Type | THERMO-FLEX – Constant Wattage, Series Resistance Type: TT/TC/SS |
| 2 | Construction | Primary Insulation: P.T.F.E. Secondary Insulation: Glass-Fibre Sheathing: SS 304 |
| 3 | Nominal Voltage | 230 V (Other on Request) |
| 4 | Nominal Loading | Door Heater: 22.5 W/Mtr – 35 W/Mtr |
| 5 | Design Standard | BS 6351 and IEEE 515 |
| 6 | Area Classification | Approved for use in Zone II Area, with gases having classification T1 to T5, as per IS 5571:1979 |
| 7 | Dimensions (Cross Section) | Door Heater: 4 mm Dia, Drain Heater: 10 × 4 mm |
| 8 | Working Temperature | -70°C to 200°C |