# THERMO-FLEX® Silicone Rubber Constant Watt Parallel Resistance Heating Cable

### Reliable Heat Tracing for Pipelines, Meters & Industrial Applications

THERMO-FLEX® Silicone Rubber Constant Power Heating Cable is designed for reliable antifreeze protection and heat preservation of pipelines, meters, tanks, and industrial equipment. Manufactured using high-quality alloy heating wire and silicone rubber high-temperature insulation, the cable provides rapid, uniform heating with high thermal efficiency and excellent flexibility.

With a maximum maintained temperature of 150°C, THERMO-FLEX® heating cables are suitable for a wide range of industrial heating and frost-protection applications. Voltage, heating length, lead length, and power can be selected according to application requirements.

## Key Features

- High-temperature resistance: Suitable for demanding industrial environments.

- Rapid and uniform heating: Provides efficient heat transfer and consistent temperature distribution.

- High thermal efficiency: Direct heat generation helps achieve the required temperature quickly.

- Flexible construction: Silicone rubber construction allows easy bending and installation.

- Excellent insulation: High-quality silicone rubber provides reliable electrical and thermal insulation.

- Strong and durable: Designed for long-term industrial use.

- Customizable: Voltage, heating length, lead length, and power can be specified according to requirements.

- Factory tested: Each heating cable undergoes electrical testing, including insulation resistance and high-voltage testing.

- Cold-tail design: Integrated cold-tail section provides a practical, joint-free construction for installation.

| Parameter | 25 W/m | 40 W/m | 50 W/m |
| --- | --- | --- | --- |
| Rated Voltage | 220 V | 220 V | 220 V |
| Voltage Range | 36–240 V* | 36–240 V* | 36–240 V* |
| Output Power | 25 W/m | 40 W/m | 50 W/m |
| Maximum Heating Length | 65 m | 50 m | 44 m |
| Heating Element | Copper-Nickel Alloy Wire | Copper-Nickel Alloy Wire | Nickel-Cadmium Alloy Wire |
| Insulation Material | Silicone Rubber | Silicone Rubber | Silicone Rubber |
| Heating Element Dimension | 5 × 7 mm | 5 × 7 mm | 5 × 7 mm |
| Wire Cross-Section | 0.75 mm² | 0.75 mm² | 0.75 mm² |
| Node Spacing | 0.5 m | 0.5 m | 0.5 m |
| Maximum Maintained Temperature | Up to 150°C | Up to 150°C | Up to 150°C |
| Environmental Temperature | -60°C to +200°C | -60°C to +200°C | -60°C to +200°C |

## Available Power Options

### 25 W/m Heating Cable

- Output: 25 W/m

- Maximum recommended length: 65 m

- Ideal for general pipeline antifreeze protection and heat preservation.

### 40 W/m Heating Cable

- Output: 40 W/m

- Maximum recommended length: 50 m

- Suitable for applications requiring higher heat output.

### 50 W/m Heating Cable

- Output: 50 W/m

- Maximum recommended length: 44 m

- Designed for applications requiring increased heating capacity.

# Applications

THERMO-FLEX® constant power heating cable can be used for:

- Pipeline antifreeze protection

- Pipeline temperature maintenance

- Water and process piping

- Industrial meters and instruments

- Tanks and vessels

- Valves and fittings

- Outdoor pipelines

- Industrial process equipment

- Frost prevention systems

- Heat preservation applications

The cable is particularly useful where dependable temperature maintenance is required in cold or demanding industrial environments.

# Construction & Performance

The heating cable consists of a copper-nickel or nickel-cadmium alloy heating element, high-temperature silicone rubber insulation, and an integrated cold-tail section. The heating element is designed to generate heat efficiently when electrical power is applied, while the silicone rubber construction provides flexibility and resistance to high operating temperatures.

Its flexible structure makes the cable easy to route around pipes, meters, valves, and other equipment, helping simplify installation while maintaining close contact with the heated surface.

## Customized According to Your Requirements

THERMO-FLEX® heating cables can be configured to meet different installation and heating requirements.

Available customization includes:

- Rated voltage: 36–240 V

- Heating length

- Lead/cold-tail length

- Output power

- Cable configuration

Please provide your required voltage, heating length, operating temperature, and application so that the appropriate THERMO-FLEX® heating cable can be selected.

## Important Installation & Safety Note

For applications in explosive gas hazardous areas (Zone 1 and Zone 2), the heating cable and complete heat-tracing system must be selected, installed, and certified in accordance with the applicable hazardous-area standards and site requirements. Do not use the product in a hazardous area solely on the basis of the general product description; verify the applicable Ex certification and installation requirements for the specific model.

### THERMO-FLEX® — Efficient Heat. Reliable Protection.

Protect pipelines from freezing. Maintain process temperatures. Deliver dependable industrial heating with THERMO-FLEX® silicone rubber constant power heating cables.