# THERMO–FLEX® Hopper Heating Pads

THERMO–FLEX® Electrical Heating Pads are specially engineered for efficient, uniform, and controlled heating of industrial hoppers. Designed to closely conform to the hopper surface, these flexible heating pads provide maximum surface contact and consistent heat transfer, making them an ideal solution for applications where reliable temperature control and prevention of material buildup, carbonization, or degradation are critical.

Unlike conventional rigid or bend-type heaters, THERMO–FLEX® Hopper Heating Pads distribute heat evenly across the available hopper surface, minimizing localized hot spots. The heating elements operate in the black-heat region, closely following the surface temperature of the hopper to deliver stable and dependable heating performance.

The pads are manufactured using Nichrome wire heating elements, insulated with glass-fibre yarn and Class F varnish, and supported by a closely woven glass-cloth carrier. The complete assembly is protected between layers of glass cloth, providing excellent electrical insulation, mechanical strength, flexibility, and durability.

THERMO–FLEX® heating pads are available with suitable mounting arrangements and temperature-control systems to meet specific process requirements. They are widely used for ESP hoppers and BF hoppers where uniform surface heating is essential.

| Parameter | Specification |
| --- | --- |
| Brand | THERMO–FLEX® |
| Product | Electrical Hopper Heating Pad |
| Maximum Safe Temperature | 350 °C |
| Maximum Power Density – ESP Hopper | 0.325 kW/m² |
| Maximum Power Density – BF Hopper | 0.500 kW/m² |
| Standard Working Voltage | 230 V, Single Phase AC |
| Alternative Working Voltage | 415 V, 2-Phase AC |
| Heating Element | Nichrome Wire |
| Element Insulation | Glass-Fibre Yarn with Class F Varnish |
| Carrier / Outer Protection | Closely Woven Glass Cloth |
| Mounting Hardware | M10 × 20 mm Studs, Nuts & Washers |
| Eyelet Pitch | Approx. 200 mm |

### Key Features

- Uniform heating over the maximum available hopper surface area

- Flexible construction that closely follows the hopper profile

- Reduced risk of hot spots and material carbonization

- Controlled and consistent heat transfer

- Suitable for high-temperature industrial applications

- Durable glass-cloth construction with electrical and thermal insulation

- Easy installation using peripheral eyelets and M10 mounting studs

- Temperature can be controlled according to process requirements

- Available for ESP and BF hopper heating applications

### Typical Applications

THERMO–FLEX® Hopper Heating Pads are particularly suitable for:

- ESP Hoppers

- BF Hoppers

- Industrial material-handling hoppers requiring controlled surface heating

- Applications where prevention of material buildup and solidification is required

THERMO–FLEX® — Flexible, Uniform & Controlled Heating for Industrial Hoppers.